Antony Saleh

- About Me

- Team Pitstop

- Design Analysis Stage

- Design Solution Stage

- Prototype Iteration

- Communication

# Design Solution Stage

## Team TVC

Rice Robotics Club - URC Wheel Design

## Problem Decomposition

### Key Functions & Features

Our design must satisfy the following core functional blocks:

- Traction Interface: Must grip soft sand and rock surfaces (Grousers/Tread).

- Structural Support: Support 50kg rover load without deformation (Core/Spokes).

- Mounting Interface: Securely attach to rocker-bogie suspension (Hub).

- Impact Dampening: Absorb shock from drop tests and boulders (Flexible Material/Geometry).

## Brainstorming & Design Selection Process

### Process Overview

Our team utilized a structured brainstorming approach over two rounds.               Round 1 focused on generating a high volume of diverse ideas using the "slip method," resulting in 87 total ideas.               Round 2 targeted specific gaps identified in the first round, specifically manufacturing methods and material selection.

### Categorized Ideas

We categorized our 80+ unique ideas into five main groups: Feature Design, Alternative Concepts, Movement Modes, Materials, and Manufacturing.

#### Wheel Features (35 Ideas)

Spoke designs, grouser shapes, and rim geometries.

#### Alternative Concepts (21 Ideas)

Tank treads, walker legs, and non-circular mobility.

#### Materials (21 Ideas)

Carbon fiber, aluminum, rubber, and hybrid composites.

#### Manufacturing

CNC machining, 3D printing, and casting techniques.

## Design Selection

We narrowed down our top ideas using a Pugh Scoring Matrix. We evaluated 8 distinct concepts against our weighted Design Criteria (Traction, Durability, Replicability, Replaceability, Weight, Size, Coolness).

| Design Criteria | Weight | Rubber w/ Spikes (Winner) | Metal Skeleton | Resin + Polycarb | Metal Star |
| --- | --- | --- | --- | --- | --- |
| Traction | 25% | 5 (1.25) | 5 (1.25) | 2 (0.5) | 4 (1.0) |
| Durability | 25% | 3 (0.75) | 3 (0.75) | 4 (1.0) | 3 (0.75) |
| Replicability | 20% | 3 (0.6) | 2 (0.4) | 4 (0.8) | 3 (0.6) |
| Replaceability | 15% | 5 (0.75) | 4 (0.6) | 5 (0.75) | 3 (0.45) |
| Weight | 5% | 3 (0.15) | 3 (0.15) | 4 (0.2) | 4 (0.2) |
| Size | 5% | 5 (0.25) | 4 (0.2) | 3 (0.15) | 4 (0.2) |
| Coolness | 5% | 3 (0.15) | 4 (0.2) | 2 (0.1) | 5 (0.25) |
| TOTAL SCORE | 100% | 3.90 (Rank 1) | 3.55 (Rank 4) | 3.50 (Rank 5) | 3.45 (Rank 6) |

Winning Concept: Rubber Wheel with Metal Spikes.

This design scored highest (3.9/5.0) because it balanced superior traction (essential for the 30° slope) with high scores in replaceability. Although simpler than the "Metal Star," it offered a more reliable and manufacturable solution for the team's timeline.

## Selected Design Solution

### Annotated Concept Sketch

Figure 2: Annotated sketch highlighting the rubber tread integration with metal grousers.

### Refined Prototype

Figure 3: The physical prototype assembled for testing.

### Operation Video

[VIDEO PLACEHOLDER]

Demonstration of the wheel traversing sand and climbing obstacles.

Play Video

The video demonstrates the wheel's ability to deform slightly over obstacles (dampening impact) while the metal spikes engage with the loose terrain to provide forward thrust.

## Testing & Validation

| Test Case | Procedure | Success Criteria | Result | Pass/Fail |
| --- | --- | --- | --- | --- |
| Traction / Slope Climb | Drive up 30° incline w/ sand | No slip > 10cm | Climbed 30° with minimal slip | PASS |
| Load Test | Apply 75kg static load | No cracking/deformation | Held 75kg, minor flex | PASS |
| Drop Test | Drop from 1m onto concrete | Operational afterwards | Rim dented, still rolls | PARTIAL |

Narrative: The prototype excelled in traction tests, easily scaling the required incline. However, the drop test revealed a weakness in the rim stiffness, which sustained minor damage. This suggests that while the tread concept is solid, the core structure requires reinforcement.

## Project Status & Next Steps

### Summary

Team TVC has successfully designed and prototyped a hybrid rubber-metal wheel that meets the primary traction and weight criteria for the URC. The "Rubber with Metal Spikes" concept outperformed complex mechanical alternatives in reliability and ease of repair.

#### Limitations

- Rim Stiffness: Vulnerable to high-impact drops (as seen in testing).

- Manufacturing Time: Spikes require manual installation, which is time-consuming for 6 wheels.

#### Future Recommendations

The next team should focus on reinforcing the inner rim with a composite weave or thicker aluminum gauge to withstand the drop test. Additionally, investigating molding the spikes directly into the tread could significantly reduce manufacturing time and part count.

document.addEventListener('DOMContentLoaded', function() {          // Initialize AOS animations          AOS.init({              duration: 1000,              easing: 'ease-in-out',              once: false,              mirror: true          });      });